Testing procedure

1.attached in breadboard pdf file

2. attached schematic pdf in file

3. Line no. 12 in given code

String number = "63539768..";        //mobile number on which you want to send sms

This is to store receiver’s mobile number in variable “number” whose variable type is string.

gsm.println("AT+CMGS=\"+91"+number+"\"\r");    //assign reciver`s number

This is command line to set receiver’s mobile number in gsm module.

4.  CONSTRUCTION & TESTING:

Assemble the circuit on PCB. Insert the sim card in the slot of GSM module. Now push the button. When push button is pressed GSM will call to the respective receiver’s mobile number which we had entered in code, and GPS will get latitude, longitude and altitude coordinates and obtain the exact location on the map. that location will be sent to the receiver through the SMS with alert message. As soon as buzzer will also start and make sound for 2 seconds to get help from surrounding people.

5. Check GPS and GSM connection

The SIM800L GSM module communicates with a microcontroller using UART protocol. This GSM module supports baud rate from 1200bps to 115200bps with Auto-Baud detection.

There is a SIM socket on the back side. 2G micro SIM card will properly work. The correct SIM card insertion direction is usually engraved on the surface of the SIM socket.

- LED Status Indicators

There is an LED on the top right side of the SIM800L which indicates the status of cellular network. It’ll blink at various rates to show what state it’s in:

Blinks every 1s

It shows that the module is running but hasn’t made any connection to the cellular network yet.

Blinks every 2s

It shows that the GPRS data connection you requested is active.

Blinks every 3s

It shows that the module has made contact with the cellular network & able to send/receive voice and SMS.

To make sure the NEO-6m GPS is working properly,

1.Connect the antenna to the GPS module.

2. Solder header pins to the GPS module.

3. Connect the USB-TTL wire to the computer.

4. Connect the TTL Rx cable with the GPS Tx and the TTL Tx with the GPS Rx

To make sure signal is received or not, open the com port in Arduino IDE and set it to GPS baud rate (here 9600 is used). You probably won’t understand the data output, but if the GPS is receiving or outputting a serial signal, we will able to see some sort of “proof of life” indication.

LED Indicator

There is an LED on the NEO-6M GPS Module which indicates the status of Position Fix. It’ll blink at various rates depending on what state it’s in

No Blinking : It shows that it is searching for satellites

Blink every 1s : It shows that Position Fix is found